# DPRAM NOTES USING W2K

Address that works best is $E8000

To see who is using what addresses, look under the Control Panel->System Properties-> Hardware tab -> Device manager -> View properties by type -> look under memory.

Use the Motion Exe program to load the correct memory location for PMAC2 to use. Run Motion Exe and click setup. Choose the correct address and close out of the program. Loading may take a while. If the PMAC is in bootstrap mode, it will alert you and give you and option to get out of bootstrap mode. If you say yes, simple go back to the Motion Exe program again and set the PMAC2 address once again for changes to take effect.

Note that these notes are for W2K and will probably be very different from the setup procedure used in Windows 98.

## Abit Motorboard BIOs checksum error problem

Sometimes when restarting the machine, there will be a BIO checksum error giving a low tone beep. The computer will not continue until it is shut off and the power on reset jumper placed on the PMAC card. As of now, it appears that when the DPRAM is turned on, it is conflicting with either the operating system or the motherboard.

The quick work around for now is to turn the computer off and place a jumper on E3, the reinitialization jumper. This will bring the PMAC to its default and make it appear once again that it does not have dual port ram. The computer will then once again boot up. After booting, once may load the DPRAM again using the MotionExe Program.

The address that works best under W2k is $E8000.

Following the notes above under DPRAM NOTES USING W2K to reload the DPRAM.